AGH University of Krakow
WEAIIB, Department of Power Electronics and Energy Control Systems
Power Electronics Laboratory

-

GaN-Based High-Frequency Switched DC-DC Synchronous Buck Converter

-

Project Design Report

Prof. dr hab. inż. Robert Stala - Ekrem Turan Fırat
12.2025, Krakow, Poland

Project Purpose: Development of educational demo board with the possibility of observing by students the heating of the system elements with changes in parameters such as frequency, load, voltage and gain.

Stages 1: Circuit Parts & Block Diagram

- Vin

  - This part is just the power input of the circuit.

- Power Protection & Input Filter

  - This component protects the circuit from factors such as over/under voltage, short circuits, overcurrent, and inrush current. It also filters the input from factors such as noise.

- Input Measurement

  - This part measures the input voltage and current.

- Power Distribution

  - This part distributes the input voltage to our operating voltage levels.

- Microcontroller

  - This part controls and supervises all systems. It performs many tasks, such as measuring input/output voltage/current, switching the buck converter, controlling the display, and adjusting output variables.

- Buck Converter:

  - This part includes the buck converter topology

- Output Filter

  - This part creates the filter for the output after switching.

- Output Measurement

  - This part measures the output voltage and current.

- Vout

  - This part is just the power output of the circuit.

Stages 2: Determining Electrical Values

- UIN  = 48V

- UOUT = Adjustable

- FSw = adjustable

- L =

  - This value has a formula [1]: UOUT × UIN - UOUT∆IL × FSw × UIN

  - ΔIL=0.3 × IOUT and If our UOUT  = 12V and POUT = 100W then IOUT  = 8.33A & ΔIL  = 2.5A

    - Calculation factory of “0.3” is standard value.

  - Final calculation is L= 12V × 48V- 12V2.5A ×500 KHz ×48V =7.2μH

- CIN =

  - We need a bulk and filter capacitors for the input capacitors.

    - Bulk Capacitor Formula: C= ∆I × ∆t∆V , If our ΔI  = 2A, Δt  = 10μs, ΔV  = 1V
 C= 2A × 10μs1V = 20 μF

    - Input Filter Capacitors: It is aimed to create a wide-scale filter by adding 100 nF , 1 μF and 3.3 μF capacitors to the input filter capacitors.

- COUT =

  - This value has a formula [2]:  ∆IL8 × FSw × ∆VOUT= 2.5A8 × 500 KHz × 0.1=6.25 μF

NOTE: When selecting capacitors, MLCC X7R low tolerance capacitor selections are priority.

Stages 3: Component Selections

- Power Protection Parts

We need a power protection part for our circuit against dangerous situations such as short circuit and overvoltage, so we will use the TPS2492PWR from Texas Instruments eFuse IC in this circuit.

  - IC Calculations

Figure [1] Typical Application Schematic

    - ILIM=50 mVRSENSE , If our current limit value is 10A, then RSENSE value will be 5 mOhm.

    - Our threshold voltage value for the UVEN and OV pins is 1.35V, so if our supply voltage range is 44V up to 52V

      - UVEN =44V×R2+R3R1+R2+R3 , OV =52V×R3R1+R2+R3 and OV = UVEN = 1.35V

      - Now our resistance values are R1 = 374 kOhm , R2 = 1.82 kOhm , R3 = 10 kOhm

    - VPROG= PLIM10 × ILIM , We need to apply a value bigger than 0.4 V to this pin, and when we calculate, our result will be 1.2 mV. Therefore, we can't use this pin. We need to connect PROG and VREF with a 47 kOhm resistor. (In this case, the PLIM value is our power lost in the MOSFET, not the total power loss of the circuit.)

    - CT=36 μA4.1V ×tON×1+ CO-TOL+ CT-TOL & tON= CO × VVCCMAXILIM

        - CT-TOL = Timer Capacitor Tolerance

        - CO-TOL = Output Capacitor Tolerance

        - tON=200 μF × 48V10A =0.96 ms

        - CT=36 μA4.1V ×0.96 ms×1+ 0.2+ 0.1=11nF

Texas Instruments - F28P650DK6PZPR

    - If we choose this MCU, It supports the high frequency PWM resolution. We can change the frequency and duty cycle with small movements.

    - Also this MCU has a two core. We can easily control the screen with the other core.

    - This MCU has a lots off PWM and ADC pins.

    - Price is 14.71€ per unite / 10.10€ per unite (100 Quantity)

- GaNFET:

  - The EPC2306, EPC2307, EPC2308, and EPC2367 were found to meet the desired range and specifications. Based on differences such as RDSon and size, it was decided to use EPC2367.

    - VDS = 100V

    - RDS(on) = 1.2 mOhm

    - ID = 101A, Pulsed 420A

    - 3.3 * 3.3

- Temperature Sensor:

  - Texas Instruments - TMP119AIYBGR

    - It has high sensitivity and small size.

- Current Measurement:

  - Current Sense Amplifier: TI INA240

- Power Protection:

  - TPS2492PWR

References

[1], [2]	B. Hauke, “Basic Calculation of a Buck Converter’s Power Stage,” Dec. 2011. [Online]. Available: www.ti.com