Prof. dr hab. inż. Robert Stala	Krakow, 24.10.2025
AGH University of Krakow
Department of Power Electronics and Energy Control Systems

Ekrem Turan Fırat
AGH University of Krakow

Designing with GaN – Miniaturization and Heating of High Frequency Converter

-

First Project Design Report

Project Purpose: Development of educational demo board with the possibility of observing by students the heating of the system elements with changes in parameters such as frequency, load, voltage and gain.

Stages 1: Determining Electrical Values

- UIN  = 48V

- UOUT = Adjustable

- FSw = adjustable

- L =

  - This value has a formula [1]: UOUT × UIN - UOUT∆IL × FSw × UIN

  - ΔIL=0.3 × IOUT and If our UOUT  = 12V and POUT = 100W then IOUT  = 8.33A & ΔIL  = 2.5A

    - Calculation factory of “0.3” is standard value.

  - Final calculation is L= 12V × 48V- 12V2.5A ×500 KHz ×48V =7.2μH

Stages 2: Component Selections

- MCU:

  - Texas Instruments - F28P650DK6PZPR

    - If we choose this MCU, It supports the high frequency PWM resolution. We can change the frequency and duty cycle with small movements.

    - Also this MCU has a two core. We can easily control the screen with the other core.

    - This MCU has a lots off PWM and ADC pins.

    - Price is 14.71€ per unite / 10.10€ per unite (100 Quantity)

- GaNFET:

  - These products are integrated power stages, meaning they have a half-bridge configuration and gate driver.

    - Texas Instruments - LMG5200

      - 80V

      - 10A

      - Max. 10 MHz Switching

      - Typical RDS(ON) = 15 mOhm

    - EPC - EPC23104

      - 100V

      - 15A

      - Max. 3 MHz Switching

      - Typical RDS(ON) = 8.7 mOhm

    - EPC - EPC2152

      - 80V

      - 15A

      - Max. 3 MHz Switching

      - Typical RDS(ON) = 10 mOhm

- Temperature Sensor:

  - Texas Instruments - TMP119AIYBGR

    - It has high sensitivity and small size.

References

[1]	B. Hauke, “Basic Calculation of a Buck Converter’s Power Stage,” Dec. 2011. [Online]. Available: www.ti.com